# PRODUCT SPECIFICATION

TYPE:       LED TV CONTROL BOARD

MODEL:    CV352-U42

| Approved by Huiguan |  | Approved by Customer |  |
| --- | --- | --- | --- |
| Prepared by: |  | Name: |  |
| Checked by: |  | Comments: |  |
| Approved by: |  | Approved by: |  |
| Date: |  | Date: |  |
| Company’s Seal: |  |  |  |

We would like you to put your signature to state your approval of the specification, and send it back to us.

ITEM

1.         DESCRIPTION 	 3

2.         FEATURES 	 3

3.        CHARACTERISTICS 	 3

4.        FUNCTION CONFIGRATION 	 5

5.         MEDIA FORMAT MODE 	 6

6.        ELECTRICAL CHARACTERISTICS 	7

7.        PROTECTION CAPABILITY 	 8

8.        INTERNATIONAL STANDARDS 	 8

9.        INTERFACE DEFINITION 	 10

10.       CONFIGURATION 	 18

11.      APPLICATION REQUIREMENT 	 19

12.       EXCURSUS 	20

13.      TRADEMARK INFORMATION 	20

History of Revision

| No. | PCBA Rev. | Details of Change | Author | Issue Date |
| --- | --- | --- | --- | --- |
| 00 | 12 | Initial Release |  | July 16, 2025 |

# 1. DESCRIPTION

CV352-U42 is a digital and analog TV control board, It can receive the PAL/SECAM/NTSCanalogue television signals and DVB-C/T/T2/S2/ATSC/ISDB digital television signals, It is suitable for the market in Europe,South america,South america,Asia pacific market.

CV352-U42是数模一体电视控制板，可接收PAL/SECAM/NTSC制式的模拟电视信号和DVB-C/T/T2/S2/ATSC/ISDB制式的数字电视信号。适用于欧洲，南美，北美，亚太市场。

CV352-U42’s power part is an energy-efficient ultrathin DC-line switching power supply unit, with backlight power under 45W, The main promotion backlight voltage is between 30V-110V,And single backlight output. It fits to 32〞 and 42〞LED backlight panel.

CV352-U42采用的高效率开关电源，匹配背光功率45W以下，匹配背光电压为30V-110V，单路背光输出,支持32寸和42寸LED背光屏。

# 2. FEATURES

HDMI1、HDMI2、HDMI3 supports CEC, HDMI1 support ARC.

HDMI1、HDMI2、HDMI3 支持CEC，HDMI1支持ARC。

The HDMI supports 1.4b, the HDCP supports 1.4.

HDMI 支持1.4b ，HDCP支持1.4。

Supports USB update and playing multimedia and JPEG.

支持USB升级软件，播放多媒体和JPEG。

# 3. CHARACTERISTICS

| PANEL | Type | TFT-LCD/LED |
| --- | --- | --- |
|  | Resolution | Max 1920X1080 |
|  | Interface | Single /Double LVDS |
|  | Voltage | 12V |
| ATV | Receiving range | 47.25MHz – 867.25MHz |
|  | Input Impedance | 75Ω |
|  | Video System | PAL, NTSC,SECAM |
|  | Sound System | BG, DK, I, L/L’ |
|  |  | NICAM/FM/AM/A2 |
| DTV | Receiving range | DVB-T/T2: VHF (42MHz – 230MHz), UHF (474MHz - 862MHz) |
|  |  | S2(950MHz – 2150MHz) |
|  |  | ATSC:CH2~CH69 |
|  |  | ISDB-T(177MHz – 803MHz) |

|  | Input Impedance | 75 Ω |  |  |
| --- | --- | --- | --- | --- |
|  | Video System | DVB-T/T2/C/S2:MPEG-2 MP@ML, MPEG-2 MP@HL, H.264, H.265:Main/Main-10 profile @L4.1,Support 8-bit/10bit color depth |  |  |
|  |  | ATSC:MPEG 2 MP@HL |  |  |
|  |  | ISDB：MPEG-2 MP@HL |  |  |
|  | Bandwidth | DVB-T/T2/C/S2:7MHz/8MHz |  |  |
|  |  | ATSC/ISDB:6MHz |  |  |
|  | Modulation | DVB-T: COFDM 2/8K, QPSK, 16QAM, 64QAM |  |  |
|  |  | DVB-T2: QPSK, 16QAM, 64QAM, 256QAM |  |  |
|  |  | DVB-C: 16QAM, 32QAM, 64QAM, 128QAM, 256QAM |  |  |
|  |  | DVB-S2: QPSK, 8PSK |  |  |
|  |  | ATSC:8VSB QAM:64QAM, 256QAM |  |  |
|  |  | ISDB-T:DQPSK/QPSK QAM:16QAM, 64QAM |  |  |
|  | Sound System | DVB-T/T2/C/S2:MPEG-1 Layer 1/2, MPEG-2 Layer 2, DD, DD+ |  |  |
|  |  | ATSC:MPEG 1/ layer 1/AC-3 |  |  |
|  |  | ISDB：MPEG-1 Layer 1/2, MPEG-2 Layer 2, DD, DD+ |  |  |
| AV | Color system | PAL/NTSC/ SECAM |  |  |
|  | Video Level | 1Vp-p@75ohm |  |  |
| HDMI | Format | 480i, 480p, 576i, 576p, 720p, 1080i, 1080p |  |  |
| Processor | CPU | A53 4 Core @max 1GHz |  |  |
|  | GPU | Mali450MP2 Dual Core |  |  |
|  | O/S | Android 14 |  |  |
|  | DDR | External *512MB/1GB (Byte) DDR3 |  |  |
|  | eMMC Flash | *4GB/8GB (Byte) |  |  |
| SUPPORT WLAN | ON BOARD CHIPSET | ATBM6032；AIC8800 WIFI Module |  |  |
|  | Max Data Rate | 150Mbps |  |  |
|  | Operating Frequency | 2.4GHz |  |  |
|  | Communications Standard | IEEE 802.11b/g/n |  |  |
|  | External support list | / |  | / |
| RJ45 NETWORK | 10/100M auto-identification and DHCP |  |  |  |
| AUDIO INPUT | CVBS Audio | Audio Level | 0.2----2 Vrms |  |
|  | Component Audio |  |  |  |
| AUDIO OUTPUT | Frequency response | 100Hz-15KHz @±3dB（1KHz -12dB reference signal） |  |  |
|  | Max Output Power | 2*8W @ 8ohm THD+N<10%@1KHz （Audio Input:0.5Vrms） |  |  |
| Keyboard | POWER |  |  |  |
| Standby | ≤ 0.5W |  |  |  |

The optional items are marked with―*Π.

# 4. FUNCTION CONFIGRATION

|  | 配置一 (25W ，32 寸) / OPTION 1（25W ，32”） 功能: 1GB DDR (2 颗 512MB)，8GB EMMC, 1T1RWIFI(ON BOARD)，内置 PCB WiFi 天线，不带 DB,带 ARC，带 CEC, 12V 屏供电，带 LVDS 输出，RF 螺纹头，RF 直头，不带天线馈电，内置 KEY(出板 2.5mm)，内置 IR/LED 电源: AC 输入 90V 到 264V ，背光电压范围在 30V~85V 之间，背光插座 2PIN2.0+3PIN2.0 立式并联不带卡扣, 不带 EMI FUNCTIONS: 1GB DDR,8GB EMMC, 1T1R WIFI(ON BOARD) ，with PCB WiFi ANT ，WITHOUT DD+,with ARC ，with CEC, 12V PANEL POWER ，with LVDS OUTPUT， RF F connector ，RF IEC connector ，WITHOUT ANT POWER， INTERNAL KEY/IR/LED POWER ： AC INPUT 90V-264V, BACKLIGHT VOLTAGE 30V-85V, BACKLIGHT CONNECTOR 2PIN 2.0+3PIN2.0 VERTICAL PARALLEL ，WITHOUT EMI |
| --- | --- |
| SIDE VIEW |  |
| RJ45, TV1(DVB-S2),TV2(DVB-C/T/T2/ATSC/ISDB),MINI AV,COAX, USB(双层), HDMI1(ARC), HDMI2（UART）, HDMI3, EARPHONE, CI. |  |
|  | 配置二 (45W ，42 寸) / OPTION 2（45W ，42”） 功能: 1GB DDR (2 颗 512MB)，8GB EMMC, 1T1RWIFI(ON BOARD)，内置 PCB WiFi 天线，不带 DB,带 ARC，带 CEC, 12V 屏供电，带 LVDS 输出，RF 螺纹头，RF 直头，不带天线馈电，内置 KEY(出板 2.5mm)，内置 IR/LED 电源: AC 输入 90V 到 264V ， 背光电压范围在 35V-110V 之间，背光插座 2PIN2.0+3PIN2.0 立式并联不带卡扣, 不带 EMI FUNCTIONS: 1GB DDR,8GB EMMC, 1T1R WIFI(ON BOARD)，with PCB WiFi ANT，WITHOUT DD+,with ARC，with CEC, 12V PANEL POWER，with LVDS OUTPUT， RF F connector，RF IEC connector，WITHOUT ANT POWER， INTERNAL KEY/IR/LED POWER：AC INPUT 90V-264V, BACKLIGHT VOLTAGE 35V-110V, BACKLIGHT CONNECTOR 2PIN 2.0+3PIN2.0 VERTICAL PARALLEL，WITHOUT EMI |
| SIDE VIEW |  |
| RJ45, TV1(DVB-S2),TV2(DVB-C/T/T2/ATSC/ISDB),MINI AV,COAX, USB(双层), HDMI1(ARC), HDMI2（UART）, HDMI3, EARPHONE, CI. |  |

*REQUIREMENT  OF MINI AV WIRE HARNESS

5. MEDIA FORMAT MODE

| Media | Codec format | Max code rate | Specification | profile | File Extension |
| --- | --- | --- | --- | --- | --- |
| MOVIE | AVS | 20 Mbps | 1920x1088@60 FPS | baseline@level6.0 | TS |
|  | AVS+ | 100 Mbps | 1920x1088@60 FPS | broadcasting @level6.0.3.08.60 |  |
|  | H264 | 50 Mbps | 1920x1080@60 FPS | BP，MP，HP @level4.2 | AVI，TS，MPEG，MP4，MOV， M2TS，MKV，FLV，MTS， M4V，ASF，F4V，3GP，3G2， TRP |
|  | H265 | 50 Mbps | 1920x1080@60 FPS 10bit | Main/Main-10@L4 .1，high tier | TS，MP4，MOV，MKV |
|  | MPEG-1 | 80 Mbps | 1920x1088@30 FPS | MP@LL，ML，HL | AVI，MPEG，DAT， M1V， M2V |
|  | MPEG-2 | 80 Mbps | 1920x1088@30 FPS | MP@LL，ML，HL | AVI，TS，MPEG，M2TS，M2T， VOB，M2V， TP， TRP，M2P |
|  | MPEG-4 | 50 Mbps | 1920x1088@30 FPS | SP@L0-3， ASP@L0-5 | AVI，TS，MP4，MOV， ASF， 3GP，3G2 |
|  | VC1 | 45 Mbps | 1920x1088@30 FPS | AP@L0～3， SP@LL，ML， MP@LL，ML，HL | AVI，TS，M2TS，MKV，ASF |
|  | VP6 | 40 Mbps | 1920x1088@30 FPS | - | AVI，FLV |
|  | VP8 | 40 Mbps | 1920x1088@30 FPS | - | WEBM |
|  | VP9 | 30 Mbps | 1920x1080@60 FPS 10bit | Profile 2（only 8bit/10bits） | WEBM |
|  | MJPEG | 150 Mbps | 1920x1088@30 FPS | - | AVI，MOV |
| SOUND | AAC | 24 kbps ~ 576 kbps | 8 kHz ~ 96 kHz | - | MKA，M4A，AVI，MP4，MOV， TS |
|  | AC3 | 32 kbps ~ 640 kbps | 32 kHz，44.1 kHz， 48 kHz | - | MKA，AVI，MP4，TS |
|  | ADPCM | 384 kbps | 8 kHz ~ 44.1 kHz | - | WAV |
|  | APE | max 1411 kbps | max 48 kHz | - | APE |
|  | DTS | max 6Mbps | max 48 kHz | - | MKA，AVI，MP4 |
|  | DTS HD | max 24.567Mbps | max 192 kHz | - | MKA，MP4 |
|  | EAC3 | 32 ~ 6Mbps | 32 kHz，44.1 kHz， 48 kHz | - | TS，MP4 |
|  | FLAC | max 1411 kbps | max 192 kHz | - | MKA，OGG，MOV |
|  | MID | max 320 kbps | max 48 kHz | - | MID |
|  | MPEG | 32 kbps ~ 320 | 8 kHz ~ 48 kHz | - | MP3，MKA，AVI，MP4，MPG， |

|  |  | kbps |  |  | OGG，MOV |
| --- | --- | --- | --- | --- | --- |
|  | PCM | max 320 kbps | 8 kHz ~ 192 kHz | - | WAV，MKA，AVI，OGG，MOV |
|  | TrueHD | max 18Mbps | 2 kHz，44.1 kHz，48 kHz，96 kHz | - | TS |
|  | Vorbis | max 256 kbps | 8 kHz ~ 48 kHz | - | OGG，MKA，MOV |
| PICTURE | png | - | 8K*8K | - | - |
|  | jpeg | - | baseline 8K*8K, progressive 4K*4K | - | - |
|  | bmp | - | 8K*8K | - | - |
|  | gif | - | 8K*8K | - | - |
| Note: The product includes the following optional patent technology: DD, DD+, DTS, WMA, WMV, RM, RMVB etc. Related patent license should be provided if you need to use these patents. The above patent using authorization is subject to our actual confirmation. (本产品包括如下可选的专利技术：DD, DD+, DTS,WMA,WMV,RM,RMVB 等，如您需要开通这些功能，请提前提供相关专利许可给到我司。以上专利使用授权以我司实际确认为准。) |  |  |  |  |  |

6. ELECTRICAL CHARACTERISTICS

6.1 Input Electrical Specifications

| Input | Minimum | Nominal | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Voltage | 90 | 240 | 264 | Vac |
| Current | —— | —— | 2.0 | A |
| Frequency | 47 | 50/60 | 63 | Hz |
| Efficiency |  | ≥80%min(Full load with input AC voltage of 110Vac @ 60Hz and 240Vac @ 50Hz) |  |  |
| Standby Power Consumption |  | ≦0.5W At 240Vac input and no load condition |  |  |
| Inrush current |  | Cannot be damaged components |  |  |
| Leakage Current |  | Less Than 0.35mA, 240Vac input |  |  |
| Input Fuse |  | T3.15AL/250VAC |  |  |

6.2 LED Driver Power Output Connector&output characteristics PJ22/PJ24: 2/2PIN-2.0

| NO. | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | LED+ | LED+ Power Supply |
| 2 | LED- | LED- Power Supply |

PJ23/PJ25: 2/3PIN-2.0

| NO. | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | LED+ | LED+ Power Supply |
| 2 | LED- | LED- Power Supply |
| 3 | LED- | LED- Power Supply |

PCON1: AC INPUT CONNECTOR

| NO. | SYMBOL | DESCRIPTION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | L | LIVE（火线） |  |  |  |  |  |
| 2 | N | NEUTRAL（零线） |  |  |  |  |  |

7. PROTECTION CAPABILITY

LED Short Protection: （LED 短路保护）

Auto restart (自动恢复)

LED Open Protection: （LED 开路保护）

Shut down or auto restart（关断或自动恢复）

Fuse protection（保险丝过流保护）

The Fuse inside the power supply shall open when the AC input current is over the rated current of fuse. This Fuse protection will cause switching power supply to fail.

（当 AC 输入电流超过电源内置保险丝的额定电流时，保险丝必须熔断，呈开路状态，保险丝保护启动后电源将不能启动。）

8. INTERNATIONAL STANDARDS

EMI standards（EMI 标准）

The power supply shall compliance with the following radio disturbance Criterion.

该电源应符合下列无线电干扰标准。

Sound and television broadcast receivers and associated equipment

| EN55013 | Sound and television broadcast receivers and associated equipment radio disturbance characteristics limits and methods of measurement |
| --- | --- |
| GB13837 | 声音和电视广播接收机及有关设备无线电干扰特性限值和测试方法 |
| FCC CFR 47 Part 15 subpart B 美国联邦通信法规第47 卷 15 章内无意式的辐射器材的相关规定 |  |

EMS standards（EMS 标准）

Electrostatic Discharge Immunity Test: IEC-1000-4-2   8KV, Criteria B

（防静电测试标准：IEC-1000-4-2   touch discharge ±4KV,air discharge ±8KV, Criteria B）

EFT/Burst Immunity Test: IEC-1000-4-4   1KV, Criteria B

（快速脉冲群测试标准：IEC-1000-4-4   1KV, Criteria B）

Safety Compliance（安规依据）

Design to meet : IEC60950 / 2005; IEC60065

（设计依据：IEC60950 / 2005; IEC60065）

Isolation（绝缘性能）

Insulation resistance（绝缘阻抗）

| Input To Output | DC500V 50MΩmin (at room temperature) |
| --- | --- |
| Input To PG | DC500V 50MΩmin (at room temperature) |

Insulation voltage（绝缘耐压）

| Input To Output | 3000Vac 50Hz 1minute ≤10mA |
| --- | --- |
| Input To PG | 1500Vac 50Hz 1minute ≤10mA |

# 9. INTERFACE DEFINITION

The picture is for a reference only,the actual item is the standard.

The optional connectors and terminals are marked with)*Π.

| JAGE1: | 12V AGING TEST CONNECTOR |
| --- | --- |
| JL1: | LVDS CONNECTOR |
| JA1: | SPEAKE CONNECTOR |
| *JK2: | KEY&IR CONNECTOR |
| PJ22/*PJ23/ *PJ24/PJ25 | LED BLACK LIGHT CONNECTOR |
| *J2/J6/JL3 | TCONLESS CONNECTOR |

All jack recognize the square pad as first pin.

JAGE1   POWER SUPPLY CONNECTOR (2PIN/2.0)

| NO. | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | +12V | +12V Power Supply |
| 2 | GND | Ground |

JL1 LVDS INTERFACE(2×15PIN/2.0)（LVDS 插针接口）

| NO. | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | VCC | Power Supply for Panel |
| 2 | VCC |  |
| 3 | VCC |  |
| 4 | GND | Ground |
| 5 | GND |  |
| 6 | NC | NC |
| 7 | RXO0- | LVDS ODD 0- Signal |
| 8 | RXO0+ | LVDS ODD 0+ Signal |
| 9 | RXO1- | LVDS ODD 1- Signal |
| 10 | RXO1+ | LVDS ODD 1+ Signal |
| 11 | RXO2- | LVDS ODD 2- Signal |
| 12 | RXO2+ | LVDS ODD 2+ Signal |
| 13 | GND | Ground |
| 14 | GND |  |
| 15 | RXOC- | LVDS ODD Clock- Signal |
| 16 | RXOC+ | LVDS ODD Clock+ Signal |
| 17 | RXO3- | LVDS ODD 3- Signal |
| 18 | RXO3+ | LVDS ODD 3+ Signal |
| 19 | RXE0- | LVDS EVEN 0- Signal |
| 20 | RXE0+ | LVDS EVEN 0+ Signal |
| 21 | RXE1- | LVDS EVEN 1- Signal |
| 22 | RXE1+ | LVDS EVEN 1+ Signal |
| 23 | RXE2- | LVDS EVEN 2- Signal |
| 24 | RXE2+ | LVDS EVEN 2+ Signal |
| 25 | GND | Ground |

| 26 | GND |  |
| --- | --- | --- |
| 27 | RXEC- | LVDS EVEN Clock- Signal |
| 28 | RXEC+ | LVDS EVEN Clock+ Signal |
| 29 | RXE3- | LVDS EVEN 3- Signal |
| 30 | RXE3+ | LVDS EVEN 3+ Signal |

NO.             SYMBOL                                                       DESCRIPTION

1

2

VCC                                                  +12V DC Power for panel

3

4

5                    NC                                                             No Connection 6

7                  GND                                                                Ground 8

9

10

NC                                                            No Connection

11

12

13                  NC                                                             No Connection

14                  NC                                                             No Connection

15                 GND                                                                Ground

16                RXEC+                                                 LVDS EVEN Clock+ Signal

17                 RXEC-                                                  LVDS EVEN Clock- Signal

18                 GND                                                                Ground

19               RXE2+                                                  LVDS EVEN 2+ Signal

20                 RXE2-                                                     LVDS EVEN 2- Signal

21               RXE1+                                                  LVDS EVEN 1+ Signal

22                 RXE1-                                                      LVDS EVEN 1- Signal

23                RXE0+                                                     LVDS EVEN 0+ Signal

24                 RXE0-                                                     LVDS EVEN 0- Signal

25                 POL2                                            Polarity control for source driver 2

26                  POL                                               Polarity control for source driver

JL3  TCONLESS INTERFACE(51PIN/0.5)（TCONLESS 接口）

| 27 | Terminate | Input signal for level shifter to pull low in blank time |
| --- | --- | --- |
| 28 | LC | Input signal for level shifter(Low frequency clock) |
| 29 | RXO3+ | LVDS ODD 3+ Signal |
| 30 | RXO3- | LVDS ODD 3- Signal |
| 31 | GND | Ground |
| 32 | RXOC+ | LVDS ODD Clock+ Signal |
| 33 | RXOC- | LVDS ODD Clock- Signal |
| 34 | GND | Ground |
| 35 | RXO2+ | LVDS ODD 2+ Signal |
| 36 | RXO2- | LVDS ODD 2- Signal |
| 37 | RXO1+ | LVDS ODD 1+ Signal |
| 38 | RXO1- | LVDS ODD 1- Signal |
| 39 | RXO0+ | LVDS ODD 0+ Signal |
| 40 | RXO0- | LVDS ODD 0- Signal |
| 41 | GND | Ground |
| 42 | STV0 | Input signal for level shifter(start pulse for GOA) |
| 43 | NC | No Connection |
| 44 |  |  |
| 45 |  |  |
| 46 | CPV2 | Input signal for level shifter(condensed clock2) |
| 47 | CPV1 | Input signal for level shifter(condensed clock1) |
| 48 | GND | Ground |
| 49 | NC | No Connection |
| 50 |  |  |
| 51 |  |  |

J2 TCONLESS INTERFACE(60PIN/0.5)（BOE_TCONLESS 接口）

| NO. | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | RXEC+ | LVDS EVEN 5- Signal |
| 3 | RXEC- | LVDS EVEN 5+ Signal |
| 4 | RXE2+ | LVDS EVEN 4- Signal |
| 5 | RXE2- | LVDS EVEN 4+ Signal |
| 6 | RXE1+ | LVDS EVEN 3- Signal |
| 7 | RXE1- | LVDS EVEN 3+ Signal |
| 8 | GND | Ground |
| 9 | RXOC+ | LVDS ODD Clock+ Signal |
| 10 | RXOC- | LVDS ODD Clock- Signal |
| 11 | GND | Ground |

| 12 | RXE2+ | LVDS EVEN 2+ Signal |
| --- | --- | --- |
| 13 | RXE2- | LVDS EVEN 2- Signal |
| 14 | RXE1+ | LVDS EVEN 1+ Signal |
| 15 | RXE1- | LVDS EVEN 1- Signal |
| 16 | RXE0+ | LVDS EVEN 0+ Signal |
| 17 | RXE0- | LVDS EVEN 0- Signal |
| 18 | GND | Ground |
| 19 | GND | Ground |
| 20 | NC | NC |
| 21 | NC | NC |
| 22 | NC | NC |
| 23 | NC | NC |
| 24 | NC | NC |
| 25 | NC | NC |
| 26 | NC | NC |
| 27 | NC | NC |
| 28 | NC | NC |
| 29 | NC | NC |
| 30 | NC | NC |
| 31 | NC | NC |
| 32 | NC | NC |
| 33 | NC | NC |
| 34 | GND | Ground |
| 35 | AVDD | Power supply for analog circuit |
| 36 | AVDD | Power supply for analog circuit |
| 37 | AVDD | Power supply for analog circuit |
| 38 | VMID | Power supply for analog circuit VMID=1/2AVDD |
| 39 | VMID | Power supply for analog circuit VMID=1/2AVDD |
| 40 | PAIRMODE | Selects mini-LVDS input mode |
| 41 | POL | Source driver polarity inversion signal |
| 42 | TP | Source driver latch signal |
| 43 | NC | NC |
| 44 | VCOM | common electrode for LCD |
| 45 | VCOM | common electrode for LCD |
| 46 | GREF | Power supply for GAMMA reference |
| 47 | VON | Positive power supply for Level shift |
| 48 | VON | Positive power supply for Level shift |
| 49 | GND | Ground |
| 50 | VOFF | Negative power supply for Level shift |
| 51 | VOFF | Negative power supply for Level shift |
| 52 | DVDD | Power supply for digital circuit. |
| 53 | DVDD | Power supply for digital circuit. |
| 54 | GND | Ground |

| 55 | NC | NC |
| --- | --- | --- |
| 56 | NC | NC |
| 57 | STV1 | STV1 input for Level shift |
| 58 | STV0 | STV0 input for Level shift |
| 59 | CPV2 | Clock2 input for Level shift |
| 60 | CPV1 | Clock1 input for Level shift |

J6 TCONLESS INTERFACE(60PIN/0.5)（HKC/AUO_TCONLESS 接口）

| NO. | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | NC | NC |
| 2 | GND | Ground |
| 3 | RXEC+ | HKC_LVDS EVEN 5- Signal/AUO_NC |
| 4 | RXEC- | HKC_LVDS EVEN 5+ Signal/AUO_NC |
| 5 | RXE2+ | HKC_LVDS EVEN 4- Signal/AUO_NC |
| 6 | RXE2- | HKC_LVDS EVEN 4+ Signal/AUO_NC |
| 7 | RXE1+ | HKC_LVDS EVEN 3- Signal/AUO_NC |
| 8 | RXE1- | HKC_LVDS EVEN 3+ Signal/AUO_NC |
| 9 | GND | Ground |
| 10 | RXOC+ | LVDS ODD Clock+ Signal |
| 11 | RXOC- | LVDS ODD Clock- Signal |
| 12 | GND | Ground |
| 13 | RXE2+ | LVDS EVEN 2+ Signal |
| 14 | RXE2- | LVDS EVEN 2- Signal |
| 15 | RXE1+ | LVDS EVEN 1+ Signal |
| 16 | RXE1- | LVDS EVEN 1- Signal |
| 17 | RXE0+ | LVDS EVEN 0+ Signal |
| 18 | RXE0- | LVDS EVEN 0- Signal |
| 19 | GND | Ground |
| 20 | NC | NC |
| 21 | WP | IIC write protect |
| 22 | SDA | IIC data signal |
| 23 | SCL | IIC clock signal |
| 24 | GND | Ground |
| 25 | NC | NC |
| 26 | NC | NC |
| 27 | VDD33 | Power supply for digital circuit |
| 28 | VDD33 | Power supply for digital circuit |
| 29 | NC | NC |
| 30 | GND | Ground |

| 31 | HAVDD | HKC_NC/AUO_Power supply for source IC |
| --- | --- | --- |
| 32 | HAVDD | HKC_NC/AUO_Power supply for source IC |
| 33 | VAA | Power supply for analog circuit |
| 34 |  |  |
| 35 |  |  |
| 36 | NC | NC |
| 37 | NC | NC |
| 38 | GND | Ground |
| 39 | GND | Ground |
| 40 | NC | NC |
| 41 | POL | Source driver polarity inversion signal |
| 42 | TP | Source driver latch signal |
| 43 | GND | Ground |
| 44 | VSS | Power voltage for GOA circuit |
| 45 | VSSQ | HKC_NC/Power voltage for GOA circuit |
| 46 | LC2 | Gout pull down circuit controlling signal |
| 47 | LC1 | Gout pull down circuit controlling signal |
| 48 | NC | NC |
| 49 | ST | GOA scan circuit start pulse |
| 50 | GND | Ground |
| 51 | VGHD | HKC_NC/AUO_ON |
| 52 | CK8 | Clock input of GOA scan circuit |
| 53 | CK7 | Clock input of GOA scan circuit |
| 54 | CK6 | Clock input of GOA scan circuit |
| 55 | CK5 | Clock input of GOA scan circuit |
| 56 | CK4 | Clock input of GOA scan circuit |
| 57 | CK3 | Clock input of GOA scan circuit |
| 58 | CK2 | Clock input of GOA scan circuit |
| 59 | CK1 | Clock input of GOA scan circuit |
| 60 | NC | NC |

JA1   SPEAKER CONNECTOR(4PIN/2.54/BTL)（喇叭输出接口）

| NO. | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | ROUT+ | Audio R+ Channel Output |
| 2 | ROUT- | Audio R- Channel Output |
| 3 | LOUT- | Audio L- Channel Output |
| 4 | LOUT+ | Audio L+ Channel Output |

*JK2   KEY&IR BOARD CONNECTOR(14PIN/2.0)（按键、遥控控制接口）

| NO. | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | K7 | EXIT |
| 3 | K6 | POWER |
| 4 | K5 | RIGHT/V+ |
| 5 | K4 | LEFT/V- |
| 6 | K3 | INPUT |
| 7 | K2 | UP/CH+ |
| 8 | K1 | DOWN/CH- |
| 9 | K0 | MENU |
| 10 | GND | Ground |
| 11 | IR | IR Receiver |
| 12 | LED G | Green Indicator |
| 13 | LED R | Red Indicator |
| 14 | +5V | +5V DC Power Supply |

# 10.     CONFIGURATION

CI

HDMI2                  HDMI3           EARPHONE

HDMI1

USB

USB

COAX

OPTICAL

MINI

AV

RJ45                 TV1(S2)                TV2

KEY

Notes:

The PCB board is thick :1.2mm

Component the tallest altitude:16.5mm Install diameter inside the bore 3.5mm

The length, width, height and aperture of tolerance: 0.2mm

25.06

板型名

(CV352-U42)

更改标记

数量

更改单号

签 名

日期

设  计

版本:1.0

纸张:A4

审  核

物料编号

第 1 页

共 1 页

批  准

未特殊说明尺寸单位为毫米

# 11.     APPLICATION REQUIREMENT

l Relative humidity ≤80%.

l Store temperature -10~+60 ℃ .

l Use temperature 0~+40 ℃ .

l Don't force, distort or disassemble the product.

l  Don’t power on before all interface has been correctly connected.

l Avoid to using long conductive wire which may affect performance and image quality.

l Authentication of assembly machine mostly depends on the matching, please use complete machine to test and confirm.

l In order to avoid the I2C-bus address conflict, please contact our hardware engineer before using the I2C interface to communicate with peripheral device.

The product has limited ESD protection. It should be placed in conductive foam to prevent electrostatic

# 12.     EXCURSUS

Keyboard and remote definition

# 13.     TRADEMARK INFORMATION

HDMI, HDMI High-Definition Multimedia Interface, HDMI trade dress and the

HDMI Logos are trademarks or registered trademarks of HDMI Licensing Administrator, Inc. in the United States and other countries.